Critical Behavior of Electrical Transport in Sea Ice

K. M. Golden1*, H. Eicken2, A. Gully1, M. Ingham3, K. A. Jones3, J. Lin1, J. Reid4, C. Sampson1 and A. P. Worby5

1University of Utah, Department of Mathematics

155 S 1400 E RM 233, Salt Lake City, UT 84112-0090 USA

2Geophysical Institute, P.O. Box 757320

University of Alaska, Fairbanks, AK 99775-7320, USA

3School of Chemical and Physical Sciences, Victoria University of Wellington

P.O. Box 600, Wellington, New Zealand

4Geoforce Pty. Ltd., 288 Victoria Rd, Perth, 6090, Australia

5Australian Antarctic Division and ACE CRC,

University of Tasmania, Private Bag 80, Hobart, 7001, Australia

*To whom correspondence should be addressed; E-mail: golden@math.utah.edu

Fluid transport through sea ice mediates a broad range of geophysical and biological processes, such as melt pond evolution and nutrient replenishment for microbial communities. However, ﬂuid ﬂow through columnar sea ice is greatly reduced for  brine volume fractions below about 5%. In two different experiments conducted in the Arctic and Antarctic, we have examined the electrical signature of this critical ﬂuid ﬂow transition. The sea-ice conductivity data are accurately explained by percolation theory, with a universal critical exponent of 2. The data also indicate marked changes in the conductivity proﬁle with the onset of surface melt ponding. Our ﬁndings lay the foundation for electromagnetic monitoring of transport phenomena in sea ice,which can help track key transitions in the state of polar sea ice and improve projections of its fate and impact on ecosystems.

Polar sea ice is a key component of Earth’s climate system, and a leading indicator of climate change (1, 2). It also hosts extensive microbial communities which sustain life in the polar oceans (1, 3). While global climate models predict declines in sea ice area and thickness, they have signiﬁcantly underestimated recent loss in Arctic summer ice extent (4).

Improving projections of the fate of Earth’s sea ice cover and its importance for polar ecosystems depends on a better understanding of key processes and feedback mechanisms. For example, the evolution of melt ponds and summer ice albedo is constrained by drainage through porous sea ice (6). It is believed that ice-albedo feedback has played a key role in the decline of summer Arctic sea ice (7). Fluid ﬂow through sea ice mediates the ice salt budget (1), convection-enhanced thermal transport (8), ocean-ice-atmosphere CO2 exchanges (9), and biomass build-up sustained by high nutrient ﬂuxes (1, 3). It also enables snow-ice formation, driven by sea water ﬂooding and freezing at the ice surface, accounting for more than a quarter of the ice produced in the Southern Ocean (10).

It has been observed that for brine volume fractions  below about 5%, columnar sea ice is effectively impermeable to vertical fluid flow, yet is increasingly permeable for  above 5% (11). For a typical bulk salinity of 5 ppt, the critical porosity c  5% corresponds to a temperature Tc  −5oC, which is known as the "rule of fives". This critical behaviour of the fluid permeability, which constrains the above processes, reflects a connectivity or percolation threshold in the brine microstructure (11–13). If the ﬂuid transport properties of sea ice can be quantitatively linked to its electrical properties, which is the aim of this paper, then a new approach can be brought to bear in monitoring the state of sea ice and its role as a barrier to heat and mass exchange. For example, it could open the door to the development of sensors enhancing existing buoy networks to provide information on key ice processes and improve integration with satellite data.

The electrical conductivity of sea ice has been studied over the past five decades (14–19). Success in relating the electrical transport properties to microstructural characteristics of sea ice, however, has been somewhat limited. In particular, there have been no observations of critical behavior in electrical properties corresponding to the important microstructural transition expressed by the rule of fives. Here we report on two types of experiments conducted on sea ice in the polar regions. In both cases we have obtained extensive data on the electrical resistivity which clearly display critical behavior at the brine percolation threshold. Our mathematical description provides a rigorous link between fluid and electrical transport in sea ice, with both properties displaying the same type of universal critical behavior, thus laying the foundation for the types of techniques referred to above.

In the Antarctic, we have pioneering measurements of the vertical resistivity of sea ice using a four probe Wenner array on extracted ice cores. In the Arctic, we have used the technique of cross-borehole DC tomography, utilizing four vertical strings of electrodes frozen into the ice (19,20). The vertical component of the resistivity is obtained indirectly from tomographic reconstructions of the horizontal and geometric mean resistivities. Although the two data sets were obtained in different ice types formed under different environmental conditions, and by quite different methods, they agree closely and are both accurately captured with the same critical exponent from lattice percolation theory used to predict fluid permeability (12). Moreover, the Arctic data was taken in late spring and shows the transformation in electrical properties associated with onset of surface melt and pond development.

The findings presented here also have implications for measuring ice thickness a key variable in tracking the ice cover’s response to changes in external forcing, such as through global warming. Not only is thickness data important in comparing climate model predictions to observed behavior, but in specifying the initial conditions necessary for long-term numerical simulations. Key methods for obtaining such data depend on the interaction of electromagnetic (EM) fields with sea ice. For example, there has been significant interest in the development of EM induction devices (18,21) mounted on ships, planes and helicopters. These techniques, and the interpretation of the data to obtain thickness information, rely on knowledge of the electrical properties of sea ice, and how they vary with depth, temperature, salinity, and ice type. The results presented here shed significant light on such issues.

During the Sea Ice Physics and Ecosystem Experiment (SIPEX) in September and October of 2007, we measured the electrical conductivity of first year Antarctic pack ice. The study area was located off the coast of East Antarctica, between 115oE and 130oE, and 64oS and 66oS. Traditional geophysical techniques using surface-based electrodes for in-situ measurement of sea ice resistivity structure, such as the Wenner array, have been of limited utility, particularly in relating electrical response to microstructural characteristics (14, 16, 17, 19, 22). During SIPEX we adapted the Wenner array to directly measure the vertical component of the anisotropic conductivity tensor * of sea ice. At 8 of the 15 ice stations along the cruise track of the Australian icebreaker Aurora Australis, we used a Wenner electrode array along sections of extracted ice cores. The array was connected to a YEW Earth Resistance Tester operating at 38 Hz and, as indicated in Figure 1 (A) and (B), the typical spacing of electrodes was 10 cm. This set-up yields the resistance along the axis of the cylindrical ice core between probes P1 and P2, corresponding to the vertical direction in situ. We obtained 26 averaged data points from 67 raw measurements. Temperature and salinity measurements allowed us to calculate a brine volume fraction profile for each core (1).

Through numerical simulations, we have shown that for the probe spacings and ice core geometry used here, the difference in measurements between the four probe array and a more standard parallel plate configuration is less than 1.5%. These findings establish the Wenner array as a viable field method for direct resistivity measurements on ice cores.

The technique of cross-borehole DC resistivity tomography (19), where ice is probed in its natural state, utilizes vertical strings of electrodes frozen into the ice. It has been shown that this method can be used to derive the horizontal component of the anisotropic resistivity profile (19). Moreover, it has recently been demonstrated that the vertical component of * can be obtained as well (20). If a minimum of four electrode strings are used, as shown in Figure 1 (C), the geometric mean of the vertical and horizontal components of * can be derived, along with the horizontal component (20), yielding the vertical component.

Measurements of the temporal variation in the resistivity structure of first year Arctic sea ice through spring warming have been made approximately 1 km off the coast of Barrow, Alaska at 71o 21/ 56.45// N, 156o 32/ 39.01// W. Electrode strings were installed in landfast first year ice in late January 2008. Cross-borehole measurements were made on 6 separate occasions between early April and mid June 2008, allowing both the horizontal and vertical components of the ice resistivity to be derived. A sea ice mass balance site and an ice core sampling program at the same location (23) provided ice temperature and salinity data, allowing the variation in resistivity structure to be correlated with brine volume fraction .

Lattice and continuum percolation theories (24) have been used to model a broad range of disordered materials where the connectedness of one phase dominates effective behavior. For example, consider the square (d = 2) or cubic (d = 3) network of bonds joining nearest neighbor sites on the integer lattice Zd. The bonds are assigned electrical conductivities 0 > 0 (open) or 0 (closed) with probabilities p and 1 − p. Groups of connected open bonds are called open clusters, and the average cluster size grows as p increases. In this model there is a critical probability pc, 0 < pc < 1, called the percolation threshold, where an infinite cluster of open bonds first appears. In d = 2, pc = 1/2, and in d = 3, pc  0.25. Typical configurations for the d = 2 square lattice above and below the threshold are shown in Figure 2.

Let *(p) be the effective conductivity of this random resistor network in the vertical direction (24). For p < pc, *(p) = 0, as shown in Figure 2 (C). For p > pc near the threshold, *(p) exhibits power law behavior,

*(p) ~ 0(p − pc) t , 	p  pc+, 			(1)

where t is the conductivity critical exponent. For lattices, t is believed to be universal, depending only on d. In d = 2, t  1.3, and in d = 3, t  2.0 (24). There is also a rigorous bound (25) that 1  t  2 in d = 2 and d = 3. Since *(p)  0 as p  pc+, the effective resistivity *(p) = 1/*(p) diverges as p  pc+, with a vertical asymptote at p = pc, as shown in Figure 2 (D). The fluid permeability *(p) corresponding to (1), where the open bonds are pipes of fluid conductivity 0/ = r02/8  and radius r0, behaves *(p) ~ 0(p − pc) e as p  pc+, with e the fluid permeability exponent and   the fluid viscosity. For lattices, it is believed that e = t (24). In the continuum, the permeability and conductivity exponents e and t can take non-universal values, and need not be equal, such as for the three dimensional Swiss cheese model (24, 26). However, for log normally distributed inclusions, as in sea ice, the behavior is universal (12, 27). Thus for sea ice, t = e  2.

In order to use percolation theory to quantitatively describe the vertical conductivity σV(), and to provide the first link between fluid and electrical transport in sea ice, we recall our result for the vertical fluid permeability kV() (12),

kV() ~ 3( - c)2 x 10-8 m2,	  φc+.			(2)

The scaling factor k0 = 3 × 10−8 is estimated using critical path analysis (24, 28). The effective behavior of media with a broad range of local conductances is dominated by a critical bottleneck conductance related to the minimal radius in a connected pathway of appropriate scale. To relate σVto kV, we use the following relation from critical path analysis (28). With rc denoting the critical radius for our centimeter scale electrical experiments,

then

kV= rc28σVσb, 							(3)

where b is the conductivity of brine, which depends on temperature T (29). By measuring

the radii of vertical pathways in X-ray tomography images (12, 13), we estimate rc (mm)

to be in the range 0.1  rc  0.2.

It is useful to consider the vertical conductivity formation factor F = σV/b, which

removes the dependence of the effective parameter on the changing conductivity of the

brine, and depends only on the pore volume fraction and geometry. In view of (1) and (3),

F() ~ F0 ( − c)2 as   φc+, where F0 = 8k0/rc2. The estimates for rc yield a range for F0 of 6  F0  24.

In order to compare our conductivity measurements with percolation theory, we must

exclude data below c  0.05, as in (12), since the theory is only valid for   > c. Unlike fluid permeability, it is unlikely that σV  0 for   < c (20). Brine channels and intergranular pore space, while not contributing to permeability at low temperatures where the cross-sectional dimensions are less than the minimum for fluid flow, may still play a role in determining a small but significant electric conductivity. It is illustrative to display this behavior in terms of the reciprocal 1/F = ρV/b, which is the vertical resistivity formation factor. In Figure 2 (E) we show the two data sets from the Arctic and Antarctic. Although the data shown all fall in the region   > c, it is clear that across the percolation threshold the decrease in 1/F is a very good approximation to a change from non-conducting to conducting regimes. By fixing the exponent t = 2 and the threshold value c = 0.05 in the above expression for F(), a statistical best fit of the data yields a value of F0  9, which lies inside our predicted range, so that

F() ~ 9 ( − 0.05)2 as   φc+.				(4)

We see that the data agree well with the theory, and that they both exhibit divergent behavior with a vertical asymptote at the percolation threshold. Moreover, in Figure 2 (F) the Antarctic data is displayed in the logarithmic variables x = log ( − 0.05) and y = log F. The line predicted by percolation theory in (4) is y = 2x + log F0, with log F0 = 0.95, F0 = 9. Critical path analysis yields the bounds 0.8  log F0  1.4, and the statistical best fit for the Antarctic data in (F) is y = 1.99x + 0.93, where 0.93 lies inside these bounds.

To model σV() over all porosities, we consider features of the brine phase present over the full range − some degree of small-scale connectivity, and self-similarity. Hierarchical models of spheres or other grains surrounded by smaller spheres, and so on, with brine in the pore spaces, were used to model kV() in (12). The simplest model yields a result of kV() = k0 3. Via (3) we obtain an Archie’s law result of F() = F0 3. A statistical best fit of our Antarctic data yields a value of F0  16, which is again in the estimated range. In Figure 3 (A), our Antarctic conductivity data is shown along with both theories, and in (B), Arctic permeability data (12) is shown with both theories.

Finally, in Figure 4 we show cross-borehole tomographic reconstructions of the vertical resistivity formation factor for Arctic sea ice. In (A), the profile was obtained before the onset of melt pond formation. The ice is cold and electrically resistive. In (B), the profile

was obtained well after a melt pond had formed. The ice is warmer and significantly more

conductive. By 16-17 June the ice had not only thinned but had ablated from the surface. The top 2 electrodes in each string were in air, and the third was in meltwater, so that the top 0.3 m of the profile are blank. Further, the very high resistivity in the next 0.1-0.2 m likely results from the fact that in this region fresh meltwater had percolated downwards, replacing the brine. This leads to values of the resistivity in this top layer of the ice roughly two to three times higher than before the formation of the meltpond. In the bottom 0.1 m or so, we also see highly conductive ice, likely due to high porosity and sea water infiltration.

It has been demonstrated in field experiments conducted in both the Arctic and Antarctic that sea ice exhibits critical behavior in its electrical transport properties at a percolation threshold. Such behavior provides the electrical signature of a key transition in fluid transport properties, known as the “rule of fives,” which determines whether or not fluid can flow through sea ice. This on−off switch for fluid flow constrains a broad range of processes which are important in the geophysics and biology of the polar regions. The phenomenon is explained theoretically using percolation theory, which provides a universal power law describing the data from both poles, as well as the first rigorous link between the fluid and electrical transport properties of sea ice. Our findings open the door to a new generation of techniques for in situ analysis and remote monitoring of transport processes which are critical to improving projections of the future trajectory of the polar ice packs.

Acknowledgements are in (30).

References and Notes

1. D. N. Thomas, G. S. Dieckmann, eds., Sea Ice, 2nd Edition (Wiley-Blackwell, Oxford,

2009).

2. M. C. Serreze, M. M. Holland, J. Stroeve, Science 315, 1533 (2007).

3. C. H. Fritsen, V. I. Lytle, S. F. Ackley, C. W. Sullivan, Science 266, 782 (1994).

4. J. Bo´e, A. Hall, X. Qu, Nature Geoscience 2, 341 (2009).

5. J. Zhang, J. Climate 20, 2515 (2007).

6. H. Eicken, T. C. Grenfell, D. K. Perovich, J. A. Richter-Menge, K. Frey, J. Geophys.

Res. (Oceans) 109, C08007.1 (2004).

7. D. K. Perovich, B. Light, H. Eicken, K. F. Jones, K. Runciman, S. V. Nghiem Geophys. Res. Lett., 34, L19505, (2007).

8. V. I. Lytle, S. F. Ackley, J. Geophys. Res. 101, 8853 (1996).

9. S. Rysgaard, J. Bendtsen, L. T. Pedersen, H. Ramløv, R. N. Glud, J. Geophys. Res.

114, C09011, doi:10.1029/2008JC005088 (2009).

10. T. Maksym, T. Markus, J. Geophys. Res. 113, C02S12, doi:10.1029/2006JC004085

(2008).

11. K. M. Golden, S. F. Ackley, V. I. Lytle, Science 282, 2238 (1998).

12. K. M. Golden, et al., Geophys. Res. Lett. 34, L16501 (6 pages and issue cover),

doi:10.1029/2007GL030447 (2007).

13. D. J. Pringle, J. E. Miner, H. Eicken, K. M. Golden, J. Geophys. Res. (Oceans) 114

(C12017) (2009).

14. K. Fujino, Y. Suzuki, Low Temp. Sci. A21, 151 (1963).

15. J. Addison, J. Appl. Phys. 40, 3105 (1969).

16. F. Thyssen, H. Kohnen, M. V. Cowan, G. W. Timco, Polarforschung 44, 117 (1974).

17. R. G. Buckley, M. P. Staines, W. H. Robinson, Cold Reg. Sci. Technol. 12, 285 (1986).

18. J. E. Reid, A. Pfaffling, A. P. Worby, J. R. Bishop, Ann. Glaciol. 44, 217 (2006).

19. M. Ingham, D. J. Pringle, H. Eicken, Cold Reg. Sci. Technol. 52, 263 (2008).

20. K. A. Jones, M. Ingham, D. J. Pringle, H. Eicken, J. Geophys. Res., in press. (2010).

21. C. Haas, Geophys. Res. Lett. 31, L09402, doi:10.1029/2007GL030447 (2004).

22. C. Sampson, K. M. Golden, A. Gully, A. P. Worby, Surface impedance tomography for

Antarctic sea ice (2010). Submitted to Deep Sea Res.

23. M. L. Druckenmiller, H. Eicken, M. A. Johnson, D. J. Pringle, C. C. Williams, Cold

Reg. Sci. Technol. 56, 61 (2009).

24. D. Stauffer, A. Aharony, Introduction to Percolation Theory, Second Edition (Taylor

and Francis Ltd., London, 1992).

25. K. Golden, Phys. Rev. Lett. 65, 2923 (1990).

26. B. I. Halperin, S. Feng, P. N. Sen, Phys. Rev. Lett. 54, 2391 (1985).

27. B. Berkowitz, I. Balberg, Transport in Porous Media 9, 275 (1992).

28. A. P. Friedman, N. A. Seaton, Water Resources Res. 34, 1703 (1998).

29. A. Stogryn, G. J. Desargant, IEEE Trans. Antennas Propagat. AP-33, 523 (1985).

30. We are grateful for the support provided by the Division of Mathematical Sciences, the

Arctic Natural Sciences Program, and the Division of Atmospheric Sciences at the US

National Science Foundation (NSF). Adam Gully was supported by the NSF Research

Experiences for Undergraduates (REU) Program and an NSF VIGRE graduate fellow-

ship, Christian Sampson was supported by the NSF REU Program, Joyce Lin was

supported by an NSF VIGRE Postdoctoral Fellowship, and Keleigh Jones by a Victoria University of Wellington postgraduate Assistantship. We also thank the Australian

Antarctic Division and the crew of the Aurora Australis for their help and support

during the SIPEX Antarctic expedition.