The electrical anisotropy of sea ice associated with the geometry and distribution of brine pockets has meant that traditional surface based geophysical techniques for in-situ measurement of the resistivity structure have been of limited usefulness1-5. It has been shown however5 that the technique of cross-borehole dc resistivity tomography, utilizing vertical strings of electrodes frozen into the ice, can be used to derive the horizontal component of the anisotropic resistivity. Although insertion of the electrode strings causes localised disturbance of the natural state of the ice, as current is passed through undisturbed ice between electrodes in separate strings, the technique still samples ice in its natural state. It has, moreover, very recently been demonstrated6-7 that, if a minimum of four electrode strings are used, such measurements may also be used to derive the geometric mean of the vertical and horizontal components of the sea ice resistivity. Allowing direct measurement of the horizontal and geometric mean resistivities, cross-borehole measurements thus allow calculation of the vertical resistivity component and resolution of the in-situ anisotropic resistivity structure. This novel development has reinvigorated the study of the resistivity structure of sea ice as a means of understanding the microstructure of sea ice and its evolution with time, temperature, salinity and brine volume fraction.

Recent measurements of the temporal variation in the resistivity structure of first year sea ice through spring warming have been made approximately 1 km off the coast of Barrow, Alaska at 71o 21' 56.45'' N, 156o 32' 39.01'' W. Electrode strings were in installed in landfast first year ice in late January 2008 and cross borehole measurements were made on 6 separate occasions between early April and mid June 2008 allowing both the horizontal and vertical component of the resistivity structure of the ice to be derived. A sea ice mass balance site operated by the University of Alaska Fairbanks at the same location8 provided ice temperature and salinity data which allow the variation of the resistivity structure to be correlated with brine volume fraction.

1. Fujino, K. and Y. Suzuki (1963), An attempt to estimate the thickness of sea ice by electrical resistivity method II, Low Temp. Sci., A21, 151–157.

2. Thyssen, F., H. Kohnen, M.V. Cowan and G.W. Timco (1974), DC Resistivity measurements on sea ice near Pond Inlet, N.W.T (Baffin Island), Polarforschung, 44, 117-126.

3. Timco, G.W. (1979), An analysis of the in-situ resistivity of sea ice in terms of its microstructure, J. Glaciol., 22, 461–471.

4. Buckley, R.G., M.P. Staines and W.H. Robinson (1986), In situ measurements of the resistivity of Antarctic sea ice, Cold Reg. Sci. Technol., 12, 285-290.

5. Ingham, M., D. Pringle, and H. Eicken (2008), Cross-borehole resistivity tomography of sea ice, Cold Reg. Sci. Technol., 52, 263-277.

6. Jones, K.A.,  M. Ingham and D.J. Pringle (2010), Sea ice microstructure: resolving anisotropic structure through cross-borehole dc resistivity tomography, submitted to J. Geophys. Res.

7. Ingham, M., K. Jones, D.J. Pringle & H. Eicken (2009), Resolution of sea ice microstructure using cross borehole resistivity tomography, Fall Meeting, American Geophysical Union, San Francisco, 14-18 December 2009.

8. Druckenmiller, M.L., H. Eicken, M.A. Johnson, D.J. Pringle, and C.C. Williams (2009), Towards an integrated coastal sea-ice observatory: System components and a case study at Barrow, Alaska, Cold Reg. Sci. Technol., 56, 61-72